LiBR Lab Isaac Lab Guide

Last Update: 12/2025

Written by: Duncan Andrews

Code by: Duncan Andrews and Jackson Habala

This document will give instructions on how to do reinforcement learning with Isaac Lab.

NOTE:

- Italicized lines are code, regular text is instructions.

- In this guide I will often write my project’s name and use my folder paths. Be sure to update accordingly for your project!

# Initial Setup: Git and Conda

This guide is adapted from:

https://isaac-sim.github.io/IsaacLab/main/source/setup/installation/pip_installation.html

You need LongFilePaths enabled on the computer. Check if it is. If not, reach out to I.T.

Download GitHub Desktop from: https://desktop.github.com/download/.  Use the default installation settings.

Go to https://github.com/isaac-sim/IsaacLab/fork and create a new fork and give it a name. For instance, mine is named DJAIsaacLab.

Open GitHub desktop and sign in to the same account you created the fork with.

Under your repositories, you should see the fork that you made. Click on it and then select clone

Change the Local Path to "C:\Users\your_username\Documents\"

Click clone. Select "For my own purposes" and continue. Do any other initializations it asks about.

Under "C:\Users\ your_username\Documents\" you should now see a folder of the git repository that you copied

Install miniconda3 from https://www.anaconda.com/download. You may need to create an account.

Open the .exe and in the Wizard, install for: "Just Me" on your computer, otherwise you need admin

Open command prompt (windows key, cmd) and on the dropdown arrow at the top, select miniconda3. Alternatively, open Anaconda Prompt directly.

# Setting Up Conda Environment

Enter the following into the terminal and accept all terms and agree to proceed by typing y or yes, whichever it asks for:

conda install git

conda create -n env_isaaclab python=3.11

conda activate env_isaaclab

If it worked properly, you should see something like "(env_isaaclab) C:\Users\dja5722>" in the anaconda terminal.

Change your directory to where your git clone is. For example:

cd C:\Users\dja5722\Documents\DJAIsaacLab

Then install Isaac Lab:

isaaclab.bat --install

Isaac has very specific dependencies, so this code should make sure that everything is the correct version:

pip install torch==2.7.0+cu128 torchvision==0.22.0+cu128 torchaudio==2.7.0+cu128 --extra-index-url https://download.pytorch.org/whl/cu128 filelock==3.13.1 fsspec==2024.6.1 markupsafe==2.1.3 networkx==3.3 sympy==1.13.3 typing_extensions==4.12.2 packaging==23.0 sentry-sdk==1.43.0 numpy==1.26.4 pillow==11.2.1 --force-reinstall

pip install "isaacsim[all,extscache]==5.0.0" --extra-index-url https://pypi.nvidia.com

pip install rsl-rl-lib --no-deps

To run Isaac Lab, enter:

isaaclab.bat

# Running Sample Simulations

On first run, you may need to accept terms. Type out yes.

Wait until it says "[INFO]: Setup complete..." in the terminal before trying to click on anything in the simulation

Blank Environment Simulation (try this one first to make sure everything works)

isaaclab.bat -p scripts\tutorials\00_sim\create_empty.py

Ant Simulation

isaaclab.bat -p scripts/reinforcement_learning/rsl_rl/train.py --task=Isaac-Ant-v0 --headless "agent.max_iterations=200" "env.scene.num_envs=256"

Anymal/Robot Dog Simulation

isaaclab.bat -p scripts/reinforcement_learning/rsl_rl/train.py --task=Isaac-Velocity-Rough-Anymal-C-v0 --headless "agent.max_iterations=1500" "env.scene.num_envs=512"

# Viewing Sample Simulation Results

There are two ways to view the simulation results. The first version listed is for playing results of a simulation you just did, and the second version is for playing from a saved file

View Ant Results

isaaclab.bat -p scripts/reinforcement_learning/rsl_rl/play.py --task=Isaac-Ant-v0

OR for a specific checkpoint REPLACE <timestamp> and <modelnumber> with that in the file

isaaclab.bat -p scripts/reinforcement_learning/rsl_rl/play.py --task=Isaac-Ant-v0 --checkpoint C:/Users/dja5722/Documents/DJAIsaacLab/logs/rsl_rl/ant_test/<timestamp>/<modelnumber>.pt

View Dog Results

isaaclab.bat -p scripts/reinforcement_learning/rsl_rl/play.py --task=Isaac-Velocity-Rough-Anymal-C-v0

OR for a specific checkpoint REPLACE <timestamp> and <modelnumber> with that in the file

isaaclab.bat -p scripts/reinforcement_learning/rsl_rl/play.py --task=Isaac-Velocity-Rough-Anymal-C-v0 --checkpoint C:/Users/dja5722/Documents/DJAIsaacLab/logs/rsl_rl/anymal_c_test/<timestamp>/<modelnumber>.pt

# Making a Model of Your Robot

First you need to make a URDF file of your robot. I used a SolidWorks URDF exporter, detailed here: https://github.com/arab-meet/3.URDF_Simulation_Workshop/blob/main/B.%20URDF/D.%20sw2urdf/export_solid_as_urdf.md

Make the y-axis the axis of rotation for all motors.

Give the parts in the URDF tool meaningful names, it will be important later.

Next, you need to create a USD file with that URDF in Isaac Sim. Follow this guide, and I will explain other steps: https://docs.isaacsim.omniverse.nvidia.com/5.0.0/robot_setup_tutorials/tutorial_intro_assemble_robot.html

Open Isaac Sim and find the URDF File and double click on it.

Select: Referenced Model, Static Base, Natural Frequency, Force, Convex Hull, and select Convert. This will create a USD file.

Open the new USD file. Select the eye icon at the top of the screen and Show By Type > Physics > Joints. This will make it easier to visualize when setting up.

In the tree on the right side, you will see all the aspects of your assembly.

Right click on root_joint and delete it.

Right click on the top item of the tree, your default prim, and Add > Physics > Articulation Root.

Then in the joints folder, select each joint and select an appropriate range.

If the range is too high, it may lead to unintended results, even if the range is feasible for the motor.

This process is very finnicky and you just have to keep trying to get it in the right plane or angle.

Use Local Position 1 to move the dotted line, default position. Put in a degree value around which axis you need to rotate it, and then click the red cone that shows up on the model to move the axis there.

Make sure that the joint corresponding to a specific body part has its 0 or neutral angle pointing toward that body.

For example, the tail joint points toward the tail body segment/motor combo.

Then you need to add physics to your robot.

Right click the top item of the tree, and Create > Physics > Physics Material

In this you can change some of the parameters like friction and density.

# Simulation Code

In the drive, you can see some folders and code that I uploaded.

You can use it as a starting point.

You will need to tune the parameters after some trials.

From here on: MATCHING CASE IS VERY IMPORTANT. YOU WILL GET ERRORS IF NOT EVERYTHING IS CORRECT.

Open VS Code and navigate to the Isaac Lab folder.

First, we will make the code for the robot initialization.

Navigate to source>isaaclab_assets_isaaclab_assets>robots

Copy over the lizard.py file and change the name to your project, keeping it lowercase.

In the new file, change all the uses of lizard to your project name, making sure to match the case: Lizard -> Hexapod, LIZARD -> HEXAPOD, etc.

Change usd_path to the directory of your USD file. For example:

usd_path=f"C:/Users/dja5722/Documents/DJAIsaacLab/LizardFiles/V1TailAssemblyUSD/V1TailAssembly.usd"

# Under rigid_props change values and add others:

disable_gravity=False,

max_depenetration_velocity=1.0,

enable_gyroscopic_forces=True,

max_angular_velocity = 5.0, #57 rev/min is max unloaded motor speed - 5.96 rad/s *@11.1 V

max_linear_velocity = 5.0,

In the same python file, change the values under init_state. Make them the exact name that your USD uses and enter the starting values in radians.

For example, in my code I have

pos=(0.0, 0.0, 0.1),

joint_pos={

#".*_leg": 0.0,

"Front": 0,

"Back": 0,

"Tail": 0,

"FR": -0.785398,

"FL": -0.785398,

"HR": -0.785398,

"HL": -0.785398,

Now we can set up the simulation code. Go to:

source> isaaclab_tasks>isaaclab_tasks>manager_based>locomotion>velocity>config

This is where you will put your project.

Make a copy of the lizard folder I uploaded and change the name to your project.

In all files in the folder, change uses of lizard to your project name, making sure to match the case: Lizard -> Hexapod, LIZARD -> HEXAPOD, etc.

You will also need to change rsl_rl_ppo_cfg.py in the agents subfolder.

# Rewards Structure

Navigate to: source>isaaclab_assets>isaaclab_tasks>isaaclab_tasks>manager_based>locomotion>velocity>velocity_env_cfg.py

Also, in source>isaaclab>isaaclab>envs>mdp>rewards.py

Use these as a reference of what parameters you can change in your configuration of your robot.

DO NOT EDIT THESE CODES!

It is a good idea to have them open in VS code for quick reference.

In the folder for your project, Go to source>isaaclab_assets>isaaclab_tasks>isaaclab_tasks>manager_based>locomotion>velocity>config>flat_env_cfg.py

Change the body names in the code to the names of the parts of your USD file. They need to match exactly.

There are a lot of variables that you can change the weights of here. If you don't want something, you can set it to 0.0

Remember to reference rewards.py and velocity_env_cfg.py to see what you can play with.

If you comment something out, it will just use a default value.

I have some rewards already in the code

# Setting Up WandB

Go to https://docs.wandb.ai/models/integrations/add-wandb-to-any-library#python and create an account. We will follow this guide.

You will also want to get an academic license.

Go to https://wandb.ai/site/pricing/ and scroll down to “Free Forever With Academic Research”

On the bottom right you should see “Academic Trial” with a box “Complete Authentication”

Click there and fill everything out

It is free, and you just need to provide your psu email, select Pennsylvania State University and fill out the remaining info. The website of the lab is: https://sites.psu.edu/libr

On the account page, you will see your API key, we will use this later

Open an Anaconda prompt and go to your environment and cd into the Isaac lab folder.

conda activate env_isaaclab

cd C:\Users\dja5722\Documents\DJAIsaacLab

Then install wandb.

pip install wandb

wandb login

Then you will paste your API key and hit enter.

Then,

set WANDB_DISABLE_SYMLINKS=true

set WANDB_ENTITY=YourUsername-penn-state

Go to: "C:\Users\xxx###\AppData\Local\miniconda3\envs\env_isaaclab\Lib\site-packages\rsl_rl\utils\wandb_utils.py"

Replace def save_model with:

def save_model(self, model_path, iter):

#wandb.save(model_path, base_path=os.path.dirname(model_path)) -- original line

import os, shutil

import wandb

# 1) Copy the checkpoint into the W&B run folder (avoids symlinks on Windows)

run = getattr(wandb, "run", None)

if run is not None:

try:

dst = os.path.join(run.dir, os.path.basename(model_path))

# copy2 keeps mtime/metadata

shutil.copy2(model_path, dst)

except Exception as e:

print(f"[WARN] W&B copy failed: {e}")

# 2) Also log it as an Artifact so it uploads reliably

try:

art = wandb.Artifact(name=f"checkpoint-{int(iter):06d}", type="model")

art.add_file(model_path, name=os.path.basename(model_path))

run.log_artifact(art)

except Exception as e:

print(f"[WARN] W&B artifact log failed: {e}")

else:

# If W&B isn't initialized, just skip silently

print("[WARN] W&B run is None; skipping model upload.")

Replace def save_file with:

def save_file(self, path, iter=None):

#wandb.save(path, base_path=os.path.dirname(path)) -- oringial line

import os, shutil

import wandb

run = getattr(wandb, "run", None)

if run is None:

print("[WARN] W&B run is None; skipping save_file upload.")

return

# Copy into the run directory (avoids Windows symlink)

try:

dst = os.path.join(run.dir, os.path.basename(path))

shutil.copy2(path, dst)

except Exception as e:

print(f"[WARN] W&B copy (save_file) failed: {e}")

# Also log as an artifact so it uploads reliably

try:

art = wandb.Artifact(name="run-files", type="run_files")

art.add_file(path, name=os.path.basename(path))

run.log_artifact(art)

except Exception as e:

print(f"[WARN] W&B artifact log (save_file) failed: {e}")

# Run and View Simulation

Finally, you are ready to run the simulation!

Reminder that you change max_iterations in rsl_rl_ppo.py

python scripts/reinforcement_learning/rsl_rl/train.py --task=Isaac-Velocity-Flat-Lizard-v0 --headless --logger=wandb --log_project_name=lizard

After it finishes starting the simulation, it will show a hyperlink. If you ctrl+click on it, you will be taken to the W&B dashboard to view the reward functions.

When training is finished you can create an .mp4 file of its performance.

python scripts/reinforcement_learning/rsl_rl/play.py --task=Isaac-Velocity-Flat-Lizard-Play-v0 --num_envs 1 --headless --video --video_length 600

The video will be saved to: YourIsaacFolder>Logs>rsl_rl>YourProject_flat>DateAndTime>Videos

To play a specific file Add to the end of the above command:

--checkpoint C:\FILEPATH\model_####.pt

Saving Joints to a .csv

In scripts>reinforcement_learning>rsl_rl change play.py to the one on the drive.

Change the name of the robot and number of joints etc.

You also need to manually change the name of the actionFileName to wherever you want to save it.

# General Isaac Guidance

Play around with the files in your project folder to get a good understanding of what can change and how.

You will learn through trying. Do your first few simulations at low number of iterations and tweak from there.

To close Isaac, do ctrl+c in the terminal, as if you were copying something.

If it asks “(Y/N)?” type Y and hit enter.

I recommend creating a .txt file with the commands you use most often and keeping that open to copy/paste. For example:

Isaac Code:

conda activate env_isaaclab

cd C:\Users\dja5722\Documents\DJAIsaacLab

isaacsim

python scripts/reinforcement_learning/rsl_rl/train.py --task=Isaac-Velocity-Flat-Lizard-v0 --headless

python scripts/reinforcement_learning/rsl_rl/play.py --task=Isaac-Velocity-Flat-Lizard-Play-v0 --num_envs 1 --headless --video --video_length 600

Good Luck!